- GPIO accessing in LPC2148:

lpc2148 has two I/O ports,

PORT0 and PORT1

PORT0  (P0.0 to P0.31) among these P0.24 ,P0.26,P0.27 are reserved pins

PORT1  (P1.16 to P1.31)

Q) How many I/O pins in LPC2148?

ANS) 45 I/O pins

- Pin functionalities are selected by using PINSEL registers(PINSEL0,PINSEL1,PINSEL2)

- By default all the pins acts as GPIO’s

- Registers to access GPIO’s in LPC2148:

PORT0:

IODIR0

IOSET0

IOCLR0

IOPIN0

PORT1:

IODIR1

IOSET1

IOCLR1

IOPIN1

Example1:

Select P0.0 as GPIO fucntionality ,set the direction as output pin and write Logic 1

and then write Logic 0 to the port pin.

ans)

- PINSEL0 =  0X00000000;//P0.0 to P0.15 AS GPIO’s

- IODIR0   =  0X00000001;//P0.0 as OUTPUT pin rest of the pins as INPUTS

- IOSET0   =  0X00000001;//write LOGIC 1 to P0.0 and no effect on the bits which are zero

- IOCLR0   =  0X00000001;//write LOGIC 0 to P0.0 and no effect on the bits which are zero

Example2 :

select P0.3 as GPIO functionality ,set the direction as output pin and write Logic 1

and then write logic 0 to the port pin.

ans)

- PINSEL0 =  0X00000000;//P0.0 TO P0.15 AS GPIO’S

- IODIR0   =  0X00000008;//P0.3 AS OUTPUT pin

- IOSET0   =  0X00000008;//Write Logic 1 to P0.3

- IOCLR0   =  0x00000008 ;//write Logic 0 to P0.3

Example3:

Select P0.2 ,P0.3,P0.5 as GPIO’s ;Set the direction of the pins as Output pins;

And write Logic 1 to the port pins and then write Logic 0 to the port pins.

Ans:

- PINSEL0 =  0X00000000;//P0.0 TO P0.15 AS GPIO’S

- IODIR0   =  0X0000002C;//P0.2,P0.3 and P0.5 AS OUTPUT pin

- IOSET0   =  0X0000002C;//Write Logic 1 to  P0.2,P0.3 and P0.5

- IOCLR0   =  0x0000002C ;//write Logic 0 to P0.2,P0.3 and P0.5